1. Turn graph on and ensure it is not in a monitoring session

2. Connect graph to PC with USB cable (Provided with graph) This should open the graph as a new drive on your PC (NS8100)

3. Find the Language and System Folders on the seismograph and replace them with the ones you received.

4. Unplug unit from PC

5. Ensure graph is fully charged or attached to charger

6. Press Help Key

7. Select “Help Information” and press “Enter

8. Press and Hold “On” and then press “Esc”

9. Release both keys

10. Unit will update ﬁrmware. Process can take 1-3 minutes.

11. As unit restarts, Verify new Firmware Version in the Help Information Screen